KYB2003H Pressure Transmitter

Description

KYB2003H is a new kind of sensor which introduces the foreign latest technology of MEMS. It has many advantages such as high overload resistance, impact resistance, good stability. This sensor is specially developed for pavement construction machinery in harsh environments and it is particularly suitable for measuring the high pressure of large hydraulic cylinder.

Application

Cranes, Excavators, Loaders, Boring machine, Rotary drilling rig, Pavers

Features

◆Wide measuring range, high measuring accuracy

◆Good long-term stability, high reliability

◆Excellent instantaneous overload resistance

◆Anti-interference ability, good moisture resistance

◆Rugged、easy to install

◆Can be customized according to the users requirements

Specifications

| Product Type | Diffused silicon pressure transmitter |
| --- | --- |
| Media | Hydraulic oil、Gas、Various liquids compatible with 304 Stainless steel |
| Range | Any range within 0~70m |
| Supply Voltage | 10～30V/5V |
| Output | 4～20mA 、0.5～4.5V |
| Error in normal temperature | ±0.5%FS |
| Overload pressure | 3 X |
| Burst Pressure | 5 X |
| Ingress protection | IP65/IP67（Waterproof plug） |
| Electrical connection | Aviation connector, Hirschmann connector, or customized |

Application Description

Torque detection of torque limiter is measuring the pressure of transformer oil of big and small cavity of automobile crane luffing cylinder through the pressure sensor, and converted into electrical signals，sent to central organization for calculation and display.

Ordering Information

| KYB2003 |  | Pressure Transmitters |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | H | construction machinery dedicated |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 代码 |  | Range |  |  |  |  |  |  |  |  | Error in normal temperature |  |  |
|  |  |  |  |  | Minimum measuring range |  |  |  |  | Maximum measuring range |  |  |  |  |  |  |
|  |  |  | 01 |  | 0～2MPa |  |  |  |  | 0～4MPa |  |  |  | ±0.5% |  |  |
|  |  |  | 02 |  | 0～5MPa |  |  |  |  | 0～7MPa |  |  |  | ±0.5% |  |  |
|  |  |  | 03 |  | 0～7MPa |  |  |  |  | 0～10MPa |  |  |  | ±0.5% |  |  |
|  |  |  | 04 |  | 0～10MPa |  |  |  |  | 0～20MPa |  |  |  | ±0.5% |  |  |
|  |  |  | 05 |  | 0～20MPa |  |  |  |  | 0～35MPa |  |  |  | ±0.5% |  |  |
|  |  |  | 06 |  | 0～35MPa |  |  |  |  | 0～60MPa |  |  |  | ±0.5% |  |  |
|  |  |  | 07 |  | 0～60MPa |  |  |  |  | 0～100MPa |  |  |  | ±0.5% |  |  |
|  |  |  |  |  | M1 |  | 4～20mA |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | V |  | 0～5V |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | V1 |  | 0.5～4.5V |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | VX |  | User defined |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | P2 |  | M20×1.5 external thread |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | P3 |  | NPT1/4 external thread |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | P4 |  | G1/4 external thread（Dedicated to more than 20 Mpa range） |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | Px |  | User defined（subject to conversion connector） |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  | C1 |  |  | Aviation connector |  |  |  |  |
|  |  |  |  |  |  |  |  |  | C2 |  |  | Cable（Standard length 1M） |  |  |  |  |
|  |  |  |  |  |  |  |  |  | C3 |  |  | Hirschmann connector |  |  |  |  |
|  |  |  |  |  |  |  |  |  | CX |  |  | User defined |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | -I |  |  |  | New type（must choosen） |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Calibration range by user |
| KYB2003 H |  |  | 01 | M1 |  | P2 |  |  |  |  | C2 |  | -I (0～1MPa) |  |  |  |

Dimension

Application Picture

Gross weight：132g

Net weight：82g

Xinhui KangYu Control Systems Engineering, Inc.

Guifeng Hightech Village, Ximen Rd., Xinhui District, Jiangmen City, Guangdong 529100, P.R. China

Tel: 86-750-6318857	 Fax: 86-750-6318900  E-mail: sale@chinakangyu.com  Website: www.chinakangyu.com