KYDM-F Series Magnetostrictive Transducers for Oil & Chemical

Product Type:

KYDM-F series anticorrosion magnetostrictive linear level transducer,

KYDM-F series flexible magnetostrictive linear level transducer,

KYDM-F series magnetostrictive linear level transducer, (hard tube)

(oil and chemical)

Description

KYDM magnetostrictive linear level transducer is a contactless

liquids level sensor designed for accurate absolute level measurement

of liquids. It can monitor the actual fluid level and temperature at the same time. Using excellent communication for monitoring and programming by EIA-RS485, data could be remote. The design and durability of the sensor make it ideal for measuring fluid levels of petroleum. The transducer provides analog output of voltage and current and digital output of Modbus. By utilizing the integral digital temperature sensor in probe, the fluid temperature could be measured as well as the level.

Applications:

Oil and chemical Scorch

Features:

◆High accuracy to meet the high requirements metering of tank level measurement occasions;

◆Stable performance, without affect of temperature and pressure changes of gas and liquid inside the tank;

◆Various output options to meet a variety of collection system requirements;

◆Intrinsic Safety Certification:ExiaIIBT5 to adapt to a variety of safe applications;

◆Modbus, which can measure various parameters of liquid level, interface, temperature;

◆Various anti-corrosive structure, to meet a wide range of oil, chlorine and other corrosive measurement environment;

◆Easy installation, no maintenance.

Specifications:

| Measured position | Modbus—1~3 position or 1~5 temperature points Analog—1~2 position |
| --- | --- |
| Operating voltage | +24VDC±10% ；±15VDC±10% |
| Outputs | 0～5VDC 0～10VDC -5～+5VDC -10～+10VDC 4～20mADC ModBus Hart |
| Range | Rigid structure：50～5000mm Flexible structure：4000~20000mm |
| Load characteristics | Current output:Load Resistance 600Ω(Max.) |
|  | Voltage output:Load current 2mA(Max.) |
| Operating Current | ＜70mA |
| Operating temperature | -40～+85 ℃ |
| Storage temperature | -40～+100 ℃ |
| Non-linearity | ＜±0.01% F.S.or 1mm max. |
| Repeatability | ＜±0.002% F.S. |
| Resolution | 16bitD/A ,0.1mm |
| Hysteresis | ＜±0.002% F.S. |
| Temperature coefficient | ＜±0.007%F.S./℃ |
| Zero range | 100%F.S. |
| full-scale range | 100%F.S. |
| Update time | depends on range,but less than 20ms |
| Probe material | A.0Cr18Ni9（304） B.316SS |
| Housing material | A.0Cr18Ni9(304) |
| Mounting | A.threaded connection B.threaded flange |
| Cable | integral cable, mating connector, integral connector |
| Sealing | Housing:IP65,Probe IP68 |
| Explosion Proof | Exd ⅡBT5,Exia ⅡCT6 |

Note: The F.S. is short for Full Scale.

Application Overview:

KYDM Magnetostrictive liquid level transducer (Oil Chemical dedicated) provides high accurate measurement of liquids level sensor designed for huge tank or vessels. It can be tailored to special length and mounting requirements for petrochemical applications. It transmits and monitors level, interface, and temperature at the same time. In addition, volume, density and weight can be work out.

Ordering Information:

For example: KYDM-FG1M910W-GB01000M1J13-W100

Notes: 1. Pls. specify product model number for correct order.

2. Just for level transducer.

Product Dimension:

ion：

Anticorrosive

flange

Anticorrosive

shell

Anticorrosive

fitting

Anticorrosive

float

Hard tube

Hard tube

Xinhui KangYu Control Systems Engineering, Inc.

Guifeng Hightech Village, Ximen Rd., Xinhui District, Jiangmen City, Guangdong 529100, P.R. China

Tel: 86-750-6318857	 Fax: 86-750-6318900  E-mail: sale@chinakangyu.com  Website: www.chinakangyu.com