KYDM-LF Series Magnetostrictive Linear Displacement Sensors

Description

KYDM-LF series Magnetostrictive Linear

Displacement Sensor (short for KYDM-LF)

is a contactless linear position sensor providing

high accurate absolute position measurement of

displacements. Contactless sensing with highest

durability, the extremely robust sensor, ideal for

continuous operation under harshest industrial

conditions is completely modular in mechanic and electronic design. Absolute output, no periodic re-calibration and maintenance, no need to re-zero. KYDM series of Magnetostrictive Linear-Displacement Sensor can support variety of signal outputs as voltage，current，pulse，SSI，ModBus，Profibus bus, and supports single sensor measuring two coaxial position. Magnetostrictive linear displacement sensor can completely replace electronic device or encoder used in injection molding machines moving parts, injection, ejection precise positioning.

Applications

Injection molding machine

Ceramic press machine

Extrusion machine

Hydraulic press

Features

Contactless Sensing with Highest Durability;

Double encapsulation ensures high operating safety and optimum EMC protection;

Absolute output, no periodic re-calibration and maintenance, no need to re-zero;

Easy installation, no maintenance.

Specifications

| Measuring range | 80~3000mm |
| --- | --- |
| Operating voltage | +24VDC±10% |
| Outputs | 0～5VDC 0～10VDC 4～20mADC SSI |
| Non-linearity | ＜±0.05% F.S. or 100um max. |
| Repeatability | ＜±0.002% F.S. |
| Resolution | 16bitD/A , 1um |
| Temperature coefficient | ＜±0.007%F.S./℃ |
| Load characteristics | Current output: Load Resistance 500Ω(Max.) |
|  | Voltage output: Load current 2mA(Max.) |
| Operating temperature | -40～+85 ℃ |
| Storage temperature | -40～+100 ℃ |
| Sealing | IP65 |

Note: The F.S. is short for Full Scale.

Application Overview:

KYDM series Magnetostrictive Linear-Displacement Sensor using non-contact magnetostrictive principle, with durability, and can be used in the injection molding machine mold, injection, transfer seat, the top of the displacement stroke detection.

Ordering Information:

Product Diagram:

Application Picture:

Xinhui KangYu Control Systems Engineering, Inc.

Guifeng Hightech Village, Ximen Rd., Xinhui District, Jiangmen City, Guangdong 529100, P.R. China

Tel: 86-750-6318857	 Fax: 86-750-6318900  E-mail: sale@chinakangyu.com  Website: www.chinakangyu.com